UNDERSTANDING SUPER STRUCTURES PRACTICAL EXERCISE #3

RESIDUAL STRESSES|BENDING CARROTS

Introduction

This is a simple practical exercise that you can perform in your kitchen using the principles associated with the bending of beams to explore the residual stress in a carrot.

Objective

To estimate the level of residual stresses in a carrot.

Apparatus

You will need a chopping board and a sharp knife to dissect your carrots; and to make some measurements, a ruler and a separate straight edge.  Some fresh, straight carrots.

Procedure

- Select carrots that are reasonably straight with a constant, round cross-section.

- Cut off the tapered end and the rounded top to form a rough straight cylinder.

- Using a large sharp knife, so that you can make a single, straight incision, cut a carrot in half longitudinally; and then, cut each half in half again longitudinally so that you have four sections each with a quarter-circle cross-section.  If your carrot contained significant residual stress, then your quarters will not be straight, unlike the original cylinder, because the slicing the carrot releases the residual stress and allows the sections to bend.

- If your section carrot does not bend then it did not contain any residual stress.  This might happen if the carrot has split during harvesting or storage, or if it has dried out.  Try another one!  An alternative cutting procedure, is to trim the carrot into a straight-sided, symmetrical beam, then cut it into longitudinal quarters as before; but now remove thin slices from one quarter such that each slice contains both core and outer material – these should bend revealing the presence of residual stress in the original carrot.

- Use the straight-edge and ruler, to measure the chord length, χ formed by each quarter, and the rise, ζ as shown in figure 1.  Measure the height of the beam, h formed the section of carrot.

- Repeat the measurements of each quarter of the carrot and use the average value in the analysis described below.

Discussion/Analysis

This can be thought of a flexural stress problem in reverse.  The carrot grows such that stress is locked into its structure, with residual stress in the core reacted by a residual stress in the outer sections which is of opposite sign.  When we disturb the three-dimensional residual stress system by slicing the carrot, we destroy the balance of residual stresses and this causes the cut section to bend due to the unbalanced stresses.  As before, stress, σ and strain, ε are related as:

σ=εE											[1]

where E is the modulus of elasticity (typically 1.36 MPa for fresh carrots).  When a beam bends, it is possible to relate the strain at a distance from the neutral axis to the radius of curvature, ρ using geometry to obtain:

Figure 1: Schematic showing chord, χ and rise, ζ dimensions, for carrot section with radius of curvature, ρ.

ε=yρ					[2]

Hence, we can estimate the unbalanced residual stresses, σ, by combining these expressions:

σ=yEρ					[3]

And, the radius of curvature can be estimated using the measurements of the arc length, χ and rise, ζ, as

1ρ=2χ22-ζ2				[4]

And, 	σ=2yEχ22-ζ2				[5]

Conclusions

What was the average residual stress that you estimated from your measurements?

By how much did your estimate of residual stress vary from carrot to carrot?  Is this variation significant compared to errors in the estimate?

What can you deduce about the sign of the residual stress in the core and outer sections for the carrot, i.e. are they in tension or compression?  Is this consistent for all carrots?

Is there a structural advantage for carrots to contain residual stress?

Thiel BL & Donald AM, In situ mechanical testing of fully hydrated carrots in the environmental SEM, Annals of Botany, 82:727-733, 1998.